EXPERIMENT – 1

Analysis, Development and Verification of 8-bit and 16-bit Arithmetic Operations Using Assembly Language Program

# Objective

A. To develop and verify 8-bit addition and subtraction using Intel 8-bit microprocessor assembly language programming.

B. To design and verify 16-bit addition and subtraction using Intel 8-bit microprocessor assembly language programming.

C. To implement and test 8-bit multiplication and division using Intel 8-bit microprocessor assembly language programming.

D. To implement and test 16-bit multiplication and division using Intel 8-bit microprocessor assembly language programming.

# Components Required

8085 Simulator Software Kit 1.0 (or) Intel 8085 microprocessor.

# Procedure

(i) Enter the input data bytes in the specified memory locations of the 8085 simulator.

(ii) Enter the corresponding 8085 assembly language program, compile it, and execute it.

(iii) Verify the output in the specified memory locations and capture the program and output screenshots.

# A(i). 8-Bit Addition

To add two 8-bit numbers and store the sum and carry.

## Program

LDA 4100H

MOV B,A

LDA 4101H

ADD B

STA 4102H

MVI A,00H

ACI 00H

STA 4103H

HLT

## INPUT

2500H = 34H (first number)

2501H = 12H (first number)

2502H = 11H (second number)

2503H = 11H (second number)

## OUTPUT

Sum (low byte) = 45H

Sum (high byte) = 23H

## Memory Data

| Memory Address | Data (Hex) |
| --- | --- |
| 2500H | 34H |
| 2501H | 12H |
| 2502H | 11H |
| 2503H | 11H |
| 2054H | 45H (sum — low byte) |
| 2055H | 23H (sum — high byte) |

## Screenshot

# A(ii). 8-Bit Subtraction

To subtract the second 8-bit number from the first and store the difference and borrow.

## Program

LDA 4100H

MOV B,A

LDA 4101H

MOV C,A

MOV A,B

SUB C

STA 4102H

MVI A,00H

ACI 00H

STA 4103H

HLT

## INPUT

4100H = 25H (minuend)

4101H = 17H (subtrahend)

## OUTPUT

Difference = 0EH

Borrow = 00H

## Memory Data

| Memory Address | Data (Hex) |
| --- | --- |
| 4100H | 25H |
| 4101H | 17H |
| 4102H | 0EH (difference) |
| 4103H | 00H (borrow) |

Note: The carry flag after SUB indicates borrow in 8085. MVI and STA do not modify flags, so ACI 00H saves that borrow as 00H or 01H.

## Screenshot

[Insert your actual simulator screenshot after successful execution.]

# B(i). 16-Bit Addition

To add two 16-bit numbers and store the sum and carry.

## Program

LHLD 2500H

XCHG

LHLD 2502H

DAD D

SHLD 2504H

MVI A,00H

ADC A

STA 2506H

HLT

## INPUT

2500H = 34H (first number low byte)

2501H = 12H (first number high byte)

2502H = 11H (second number low byte)

2503H = 11H (second number high byte)

First number = 1234H; second number = 1111H

## OUTPUT

Result = 2345H

2504H = 45H (low byte)

2505H = 23H (high byte)

2506H = 00H (carry)

## Memory Data

| Memory Address | Data (Hex) |
| --- | --- |
| 2500H | 34H |
| 2501H | 12H |
| 2502H | 11H |
| 2503H | 11H |
| 2504H | 45H (result low) |
| 2505H | 23H (result high) |
| 2506H | 00H (carry) |

Note: LHLD and SHLD are valid 8085 instructions. Use the syntax accepted by your simulator, as in your successful run.

## Screenshot

[Insert your actual simulator screenshot after successful execution.]

# B(ii). 16-Bit Subtraction

To subtract one 16-bit number from another and store the difference.

## Program

LHLD 2500H

XCHG

LHLD 2502H

MOV A,E

SUB L

MOV L,A

MOV A,D

SBB H

MOV H,A

SHLD 2504H

HLT

## INPUT

2500H = 34H; 2501H = 12H → first number = 1234H

2502H = 11H; 2503H = 11H → second number = 1111H

## OUTPUT

Difference = 0123H

2504H = 23H (low byte)

2505H = 01H (high byte)

## Memory Data

| Memory Address | Data (Hex) |
| --- | --- |
| 2500H | 34H |
| 2501H | 12H |
| 2502H | 11H |
| 2503H | 11H |
| 2504H | 23H (result low) |
| 2505H | 01H (result high) |

## Screenshot

[Insert your actual simulator screenshot after successful execution.]

# C(i). 8-Bit Multiplication

To multiply two 8-bit unsigned numbers using repeated addition; the 16-bit product is stored in memory.

## Program

LDA 4100H

MOV E,A

MVI D,00H

LDA 4101H

MOV C,A

LXI H,0000H

MOV A,C

ORA A

JZ DONE

LOOP: DAD D

DCR C

JNZ LOOP

DONE: SHLD 4102H

HLT

## INPUT

4100H = 05H (multiplicand)

4101H = 04H (multiplier)

## OUTPUT

Product = 0014H (5 × 4 = 20 decimal)

4102H = 14H (low byte)

4103H = 00H (high byte)

## Memory Data

| Memory Address | Data (Hex) |
| --- | --- |
| 4100H | 05H |
| 4101H | 04H |
| 4102H | 14H (product low) |
| 4103H | 00H (product high) |

Note: If the simulator rejects labels, use its supported label format or replace labels with assembled branch addresses.

## Screenshot

[Insert your actual simulator screenshot after successful execution.]

# C(ii). 8-Bit Division

To divide an 8-bit unsigned dividend by an 8-bit unsigned divisor using repeated subtraction.

## Program

LDA 4100H

MOV B,A

LDA 4101H

MOV C,A

MVI D,00H

MOV A,C

ORA A

JZ ZERO

LOOP: MOV A,B

CMP C

JC DONE

SUB C

MOV B,A

INR D

JMP LOOP

DONE: MOV A,D

STA 4102H

MOV A,B

STA 4103H

HLT

ZERO: MVI A,FFH

STA 4102H

HLT

## INPUT

4100H = 14H (dividend = 20 decimal)

4101H = 04H (divisor = 4 decimal)

## OUTPUT

Quotient = 05H

Remainder = 00H

For divisor 00H, the program writes FFH to the quotient location as an error marker.

## Memory Data

| Memory Address | Data (Hex) |
| --- | --- |
| 4100H | 14H (dividend) |
| 4101H | 04H (divisor) |
| 4102H | 05H (quotient) |
| 4103H | 00H (remainder) |

## Screenshot

[Insert your actual simulator screenshot after successful execution.]

# D(i). 16-Bit Multiplication (16-bit × 8-bit)

To multiply a 16-bit unsigned multiplicand by an 8-bit multiplier using repeated addition. The 24-bit result is stored in three bytes.

## Program

; Multiplicand: 4100H low byte, 4101H high byte

; Multiplier:   4102H

; Product:      4103H low, 4104H high, 4105H upper byte

LHLD 4100H

XCHG

LDA 4102H

MOV C,A

LXI H,0000H

MVI A,00H

STA 4105H

MOV A,C

ORA A

JZ DONE

LOOP: DAD D

JNC NEXT

PUSH PSW

LDA 4105H

INR A

STA 4105H

POP PSW

NEXT: DCR C

JNZ LOOP

SHLD 4103H

DONE: HLT

## INPUT

4100H = 34H; 4101H = 12H → multiplicand = 1234H

4102H = 02H → multiplier = 02H

## OUTPUT

Product = 002468H

4103H = 68H (low byte)

4104H = 24H

4105H = 00H (upper byte)

## Memory Data

| Memory Address | Data (Hex) |
| --- | --- |
| 4100H | 34H |
| 4101H | 12H |
| 4102H | 02H |
| 4103H | 68H (product low) |
| 4104H | 24H (product middle) |
| 4105H | 00H (product high) |

Note: This is 16-bit multiplicand × 8-bit multiplier, not a full 16-bit × 16-bit product. Verify label and stack instruction support in the simulator.

## Screenshot

[Insert your actual simulator screenshot after successful execution.]

# D(ii). 16-Bit Division

To divide a 16-bit unsigned dividend by a non-zero 16-bit divisor using repeated subtraction. The dividend memory is updated to the remainder.

## Program

; Dividend/remainder: 4100H low, 4101H high

; Divisor:            4102H low, 4103H high

; Quotient:           4104H low, 4105H high

MVI A,00H

STA 4104H

STA 4105H

LDA 4102H

MOV B,A

LDA 4103H

ORA B

JZ ZERO

LOOP: LDA 4100H

MOV B,A

LDA 4102H

MOV C,A

MOV A,B

SUB C

STA 4106H

LDA 4101H

MOV B,A

LDA 4103H

MOV C,A

MOV A,B

SBB C

STA 4107H

JC DONE

LDA 4106H

STA 4100H

LDA 4107H

STA 4101H

LDA 4104H

INR A

STA 4104H

JNZ LOOP

LDA 4105H

INR A

STA 4105H

JMP LOOP

DONE: HLT

ZERO: HLT

## INPUT

4100H = 14H; 4101H = 00H → dividend = 0014H

4102H = 04H; 4103H = 00H → divisor = 0004H

## OUTPUT

Quotient = 0005H

Remainder = 0000H (left at 4100H–4101H)

## Memory Data

| Memory Address | Data (Hex) |
| --- | --- |
| 4100H | 00H (remainder low after execution) |
| 4101H | 00H (remainder high after execution) |
| 4102H | 04H (divisor low) |
| 4103H | 00H (divisor high) |
| 4104H | 05H (quotient low) |
| 4105H | 00H (quotient high) |

Note: This repeated-subtraction method may take many iterations for large quotients. Do not use a zero divisor. Verify it in the simulator before recording final results.

## Screenshot

[Insert your actual simulator screenshot after successful execution.]

# Result

The 8085 assembly programs demonstrate arithmetic operations on unsigned numbers. The output is verified by checking the relevant memory locations after successful execution.

# Before Submission

Compile and execute each program in the lab simulator before recording its output.

Enter input bytes at the exact memory addresses listed for that program.

Replace each screenshot placeholder with your own simulator screenshot.

If the faculty prescribes different memory addresses or a specific code format, follow the faculty’s instructions.